Optimality, Equity, and Regulatory Feasibility in Peer-to-Peer Energy Markets: A Game-Theoretic Framework for Colombian Energy Communities

Brayan S. Lopez-Mendeza; Andrés Pantojab; Germán Obandob

⁰ᵃ Master’s Program in Electronic Engineering, Universidad de Nariño, Pasto, Colombia. ᵇ Electrical Engineering Department, Universidad de Nariño, Pasto, Colombia.

bura.vent@gmail.coma; ad_pantoja@udenar.edu.co; gobando@udenar.edu.cob

Abstract

The global energy transition is accelerating the emergence of Peer-to-Peer (P2P) energy markets, in which prosumers trade electricity directly without centralized intermediaries. In Colombia, Resolution CREG 101 072/2025 establishes the current regulatory framework for community energy markets; however, its economic performance relative to other P2P mechanisms remains uncharacterized for real prosumer communities. This work proposes a P2P market model grounded in game theory and population dynamics for a community of five prosumer institutions in Pasto, Colombia, utilizing photovoltaic generation and demand data collected over a six-month period

A Stackelberg leader–follower game is formulated in which the distribution network operator acts as the leader, setting the internal P2P trading price, while prosumers act as followers, independently maximizing their individual net economic gains. Convergence to a Nash equilibrium is determined through Replicator Dynamics, a population dynamics model that captures strategic adaptation without requiring central coordination. Two regulatory benchmarks are used for comparison: CREG Resolution 174/2021 (C1 – net-metering self-consumption scheme) and the current CREG 101 072/2025 (C2 – community energy market). To analyze the robustness and performance of these market mechanisms, a sensitivity analysis is conducted by varying Grid Buying Prices (GBP) across historical values of the Colombian market. Furthermore, regulatory feasibility is assessed against the participation constraints established by CREG 101 072/2025.

Results based on the community’s average daily profiles indicate that the proposed P2P mechanism yields substantially higher net economic gains for all community members compared to both C1 and C2, demonstrating improvements across self-consumption, self-sufficiency, and benefit-distribution equity metrics. The Price of Fairness index is employed to quantify the trade-off between economic efficiency and equitable benefit allocation. Sensitivity analysis suggests that this P2P advantage remains robust across a wide range of GBPs, supporting the mechanism’s applicability under diverse market conditions. However, the regulatory feasibility assessment indicates that the current participation constraints in CREG 101 072/2025 may limit the full operation of community-scale P2P markets, as several agents exceed the established individual participation thresholds.

Ultimately, the proposed framework demonstrates the viability of Stackelberg-based P2P markets as a superior alternative to existing Colombian regulatory schemes for energy communities. These findings provide quantitative grounds for regulatory revision, specifically regarding the participation constraints in CREG 101 072/2025, and contribute methodological tools applicable to other community energy contexts throughout Latin America.

Keywords

P2P energy market; Stackelberg game theory; Replicator Dynamics; CREG 101 072/2025; prosumers.

These data were sourced from MTE, a project focused on P2P markets for energy communities in Colombia.